Protected Mode programming for the  intel386tm, intel486tm                                       	ANd pentiumtm processors 							(PART 2 of 3)

Title:		Protected Mode programming for the  						intel386, intel486 and pentium 								processors (PART 2 of 3)

Intel Ref. No: 	[7144.doc] [ ] [mfg_PB]

Faxback No:	7144

Prods. covered:	 80386, 80486, Pentium

Date:  		10/11/93

Related Info:	See Faxback documents [7143], [7145][7146]

Keywords:		Protected Mode, 386, 486, Pentium, software

FaxBACK* Service:

Quick, Simple, Easy. Available 24 hours a day!

1-800-628-2283 or (916)356-3105 (US or Canada)

+44(0)793-496646 (Europe)

Bulletin Board Services (BBS):

Latest Product Information and Support, 24 hours a day!

(916)356-3600 (US or Canada)

+44(0)793-496340 (Europe)

Download the Latest FaxBACK Catalogs and FaxFind Windows Utility

1-800-897-2536 (US or Canada)

Europe, call BBS # above and look under FaxBACK area in File Locator

Literature Hotline:

For Current Literature Guide  Order No. 210620

1-800-548-4725 (US or Canada)

Europe, contact local Intel office

* Other brands and names are the property of their respective owners.

Protected Mode programming for the  		   intel386, intel486 	ANd pentium					    processors (PART 2 of 3)

Example 4.  Exception HandlersTC "4	Exception Handlers" \F X \L 2§

KERNEL_CODE32  SEGMENT USE32 PUBLIC

ASSUME  CS:KERNEL_CODE32,DS:KWAIT_DATA32,SS:KERNEL_STACK32

EXCEPTION_BEEP:

;INPUT--CX = counter value

;            --frequency = 1.19*10^6/(counter value)

;       BX = number of beeps

MOV     AL,0B6H         ;Command: select counter 2

OUT     43H,AL          ;Write control word

MOV     AX,CX           ;Get count

OUT     42H,AL          ;Low byte of counter 2

MOV     AL,AH

OUT     42H,AL          ;High byte of counter 2

EXB1:

MOV     AL,3            ;Gate sound on

OUT     PORT_B,AL

MOV     ECX,500000H

LOOP    $               ;Delay

MOV     AL,0            ;Gate sound off

OUT     PORT_B,AL

MOV     ECX,500000H

LOOP    $               ;Delay

DEC     BX

JNZ     EXB1            ;Do again!

PJMPF32_16      RET_CODE16_SEL,RET_REAL         ;Done

EXCEPTION_HANDLER0:

;All handlers are similar--they generate a series of beeps

;Frequency increases with exception number

;Number of beeps = exception number plus 1

MOV     CX,1800H

MOV     BX,1

JMP     EXCEPTION_BEEP

EXCEPTION_HANDLER1:

MOV     CX,1700H

MOV     BX,2

JMP     EXCEPTION_BEEP

EXCEPTION_HANDLER2:

MOV     CX,1600H

MOV     BX,3

JMP     EXCEPTION_BEEP

EXCEPTION_HANDLER3:

MOV     CX,1500H

MOV     BX,4

JMP     EXCEPTION_BEEP

EXCEPTION_HANDLER4:

MOV     CX,1400H

MOV     BX,5

JMP     EXCEPTION_BEEP

EXCEPTION_HANDLER5:

MOV     CX,1300H

MOV     BX,6

JMP     EXCEPTION_BEEP

EXCEPTION_HANDLER6:

MOV     CX,1200H

MOV     BX,7

JMP     EXCEPTION_BEEP

EXCEPTION_HANDLER7:

MOV     CX,1100H

MOV     BX,8

JMP     EXCEPTION_BEEP

EXCEPTION_HANDLER8:

MOV     CX,1000H

MOV     BX,9

JMP     EXCEPTION_BEEP

EXCEPTION_HANDLER9:

MOV     CX,0F00H

MOV     BX,10

JMP     EXCEPTION_BEEP

EXCEPTION_HANDLER10:

MOV     CX,0E00H

MOV     BX,11

JMP     EXCEPTION_BEEP

EXCEPTION_HANDLER11:

MOV     CX,0D00H

MOV     BX,12

JMP     EXCEPTION_BEEP

EXCEPTION_HANDLER12:

MOV     CX,0C00H

MOV     BX,13

JMP     EXCEPTION_BEEP

EXCEPTION_HANDLER13:

MOV     CX,0B00H

MOV     BX,14

JMP     EXCEPTION_BEEP

EXCEPTION_HANDLER14:

MOV     CX,0A00H

MOV     BX,15

JMP     EXCEPTION_BEEP

EXCEPTION_HANDLER15:

MOV     CX,0900H

MOV     BX,16

JMP     EXCEPTION_BEEP

EXCEPTION_HANDLER16:

MOV     CX,0800H

MOV     BX,17

JMP     EXCEPTION_BEEP

EXCEPTION_HANDLER17:

MOV     CX,0700H

MOV     BX,18

JMP     EXCEPTION_BEEP

KERNEL_CODE32_END:

KERNEL_CODE32  ENDS

# 1	Sound Module

The example application generates sound by alternately applying current to and withdrawing current from the voice coil of the PC's speaker. The software controls the voice coil via a port of the PC's programmable peripheral controller.

## 2	Applying Protection to a Device Driver

The processor's I/O privilege level (IOPL) defines a level of privilege specifically for the execution of I/O and related instructions. Device drivers are, generally speaking, not implemented as part of the kernel, but rather as users of kernel services. They are considered less trusted than the kernel because they are subject to more frequent change. On the other hand they are critical parts of the system that should not be subject to interference from application programs. Therefore, IOPL is customarily set to a privilege level 1 or 2. In this case the IOPL is initialized to 1 in the flags field of each task's TSS.

The code in the device driver task runs at PL1, so that it can execute I/O instructions. Programs executing at privilege levels greater than 1 are not able to execute I/O. The sound specifications are placed in a global, PL1, data segment, so that the device driver code can access them, but application code cannot.

## 3	Example Code

The module that controls the sound consists of the following parts:

·	The current sound specifications. The specifications consist of the current ONPERIOD and OFFPERIOD. The full period of a sound cycle is ONPERIOD + OFFPERIOD. ONPERIOD is full period times the duty cycle; OFFPERIOD is full period times the complement of the duty cycle.

·	A procedure CHANGESOUND that changes the sound specifications. The parameters to CHANGESOUND are in units of milliseconds, which CHANGESOUND converts to timer-ticks. The procedure CHANGESOUND must be callable by application programs; therefore, it has a global, PL3 call gate.

·	The task SOUND_TASK that drives the speaker. Its logic is an infinite loop through the following steps:

1.	If ONPERIOD is nonzero, toggle speaker on (apply current to voice coil).

2.	Wait for ONPERIOD.

3.	If OFFPERIOD is nonzero, toggle speaker off (withdraw current from voice coil).

4.	Wait for OFFPERIOD.

Note that an ONPERIOD of zero causes a silence for a time interval of OFFPERIOD, and vice versa. It does not make sense for both ONPERIOD and OFFPERIOD to be zero, but this condition is harmless; the speaker state does not change, and WAIT is called repeatedly until the sound specifications change.

A period of -1 causes the application to terminate.

·	An initialization procedure CLEAR_SPEAKER_GATES which readies the speaker. The initialization procedure must be at PL0.

Example 5.  Sound Device DriverTC "5	Sound Device Driver" \F X \L 2§

;************************ SOUND module

;It contains a global code segment (_G), a local code segment (_L),

;a global data segment containing the ONPERIOD, OFFPERIOD variables,

;and two local stack segments.

SOUND_DATA32_G  SEGMENT USE32   PUBLIC

SOUND_DATA32_G_BEGIN:

ONPERIOD        DD     50       ;Defaults

OFFPERIOD       DD     50

SOUND_DATA32_G_END:

SOUND_DATA32_G        ENDS

SOUND_CODE32_G  SEGMENT USE32 PUBLIC

ASSUME  CS:SOUND_CODE32_G,DS:SOUND_DATA32_G,SS:SOUND_STACK32_L

SOUND_CODE32_G_BEGIN:

CONVERT_SOUND_PARMS    PROC

;This procedure changes the "user" parameters to clock

;tick units.

;If an error occurs, a default value is supplied.

;We assume EBX is frequency and EDI is duty_cycle

MOV     EAX,TIMER_VALUE

CMP     EBX,-1          ;Is parm = -1?

JE      QUIT_PLAYING

MUL     EBX

MOV     EBX,EAX         ;Save result

MOV     EAX,1193182     ;Actual timer input frequency

DIV     EBX             ;EAX = period in TIMER_COUNTS

CMP     EAX,0           ;Error if 0

JNE     CP1             ;OK

MOV     EAX,50          ;Use a default period

CP1:

MOV     ECX,EAX         ;Save for later use =PERIOD

MOV     EBX,EDI         ;Get duty-cycle

CMP     EBX,100         ;Is it within range?

JBE     CP2             ;Yes

MOV     EBX,50          ;No, supply default

CP2:

MUL     EBX

MOV     EBX,100         ;Form percentage

DIV     EBX             ;EAX = PERIOD*DUTY_CYCLE =ONPERIOD

SUB     ECX,EAX         ;ECX = OFFPERIOD

RET

QUIT_PLAYING:

MOV     EAX,-1          ;End of list reached

MOV     ECX,-1

RET

CONVERT_SOUND_PARMS     ENDP

CHANGESOUND     PROC    FAR

CALL    CONVERT_SOUND_PARMS  ;Convert to clock tick units

;Two parameters are returned from CONVERT_SOUND_PARMS.

;This proc uses the parameters

;to modify the ONPERIOD and OFFPERIOD variables.

;NOTE, THIS RUNS IN THE CONTEXT OF THE CALLING TASK.

;INPUT  EAX=new onperiod

;       ECX=new offperiod

PUSH    ESI

PUSH    DS

MOV     ESI,SOUND_DATA32_G_SEL  ;Load data segment

MOV     DS,SI

MOV     ONPERIOD,EAX    ;Store new data

MOV     OFFPERIOD,ECX

POP     DS

POP     ESI

RET

CHANGESOUND     ENDP

SOUND_CODE32_G_END:

SOUND_CODE32_G  ENDS

SOUND_CODE32_L  SEGMENT USE32 PUBLIC

ASSUME  CS:SOUND_CODE32_L,DS:SOUND_DATA32_G,SS:SOUND_STACK32_L

SOUND_CODE32_L_BEGIN:

SOUND_START32:

SOUND_TASK:

CMP     ONPERIOD,0

JE      ST1

IN      AL,PORT_B

OR      AL,00000010B        ;Set speaker data bit

OUT     PORT_B,AL

ST1:

MOV     EAX,ONPERIOD        ;Wait time

PCALLF32_32     WAIT_TICKS_GATE_SEL,0

IN      AL,PORT_B

AND     AL,NOT 00000010B    ;Clear speaker data bit

OUT     PORT_B,AL

MOV     EAX,OFFPERIOD       ;Wait time

PCALLF32_32     WAIT_TICKS_GATE_SEL,0

JMP     SOUND_TASK

SOUND_CODE32_L_END:

SOUND_CODE32_L  ENDS

SOUND_STACK32_L       SEGMENT

SOUND_STACK32_L_BEGIN LABEL WORD

DD      400H     DUP(0)

SOUND_STACK32_L_END LABEL WORD

SOUND_STACK32_L       ENDS

SOUND_STACK32_L_PL0       SEGMENT

SOUND_STACK32_L_PL0_BEGIN LABEL WORD

DD      400H     DUP(0)

SOUND_STACK32_L_PL0_END LABEL WORD

SOUND_STACK32_L_PL0       ENDS

# 4	The Experiment Task

The application task changes the global sound specification occasionally by calling CHANGESOUND, then waits by calling WAIT. The application code in this task runs at PL3. In our example application, this task will probably be revised often, as the experimenter tunes the parameters of the experiment.

## 5	Applying Protection to Application Programs

Application programs are typically executed as separate tasks. Their code and data segments are PL3 segments in the LDT of the task. This structure gives maximum protection among application tasks and also protects other parts of the operating system from the applications.

## 6	Example Code

In the application module, the frequency, duty cycle, and wait period until next change are stored in an assembly-language table. The code loops through this table repeatedly, calling CHANGESOUND and WAIT_MSECS to satisfy the specifications of each entry.

Example 6.  Experiment TaskTC "6	Experiment Task" \F X \L 2§

;****************** APPLICATION module

;It contains code, data, and three stacks, all defined in APP_LDT

USER_SOUND      STRUC

FREQUENCY       DD      0

DUTY_CYCLE      DD      0

DURATION        DD      0

USER_SOUND      ENDS

APP_DATA32  SEGMENT USE32   PUBLIC

APP_DATA32_BEGIN:

;The sound table uses the format FREQUENCY, DUTY_CYCLE, DURATION

;FREQUENCY in Hertz

;DUTY_CYCLE in values from 0-100

;DURATION in milliseconds

SOUND_TABLE_BEGIN     LABEL   DWORD

INCLUDE SOUNDS.ASM

SOUND_TABLE_END       LABEL   DWORD

SOUND_TABLE_PTR DD      SOUND_TABLE_BEGIN

APP_DATA32_END:

APP_DATA32        ENDS

APP_CODE32  SEGMENT USE32 PUBLIC

ASSUME  CS:APP_CODE32,DS:APP_DATA32,SS:APP_STACK32

APP_CODE32_BEGIN:

APP_START32:

APP_TASK:

;This task pulls the FREQUENCY values from the SOUND_TABLE

;and loads the sound driver's parameters.

;The SOUND_TABLE_PTR starts at the first table entry and

;is incremented each time until the end of the table. At the

;end, it wraps around to the beginning. If a -1 is found in

;the table, it simply jumps to EXIT. A -1 causes termination

;of the program.

MOV     ESI,SOUND_TABLE_PTR             ;Get pointer

MOV     EBX,[ESI].USER_SOUND.FREQUENCY  ;Get next frequency

MOV     EDI,[ESI].USER_SOUND.DUTY_CYCLE ;Get next duty cycle

PCALLF32_32    CHANGESOUND_GATE_SEL,0   ;Set new parms

MOV     ESI,SOUND_TABLE_PTR

MOV     EAX,[ESI].USER_SOUND.DURATION   ;Get duration

ADD     DWORD PTR SOUND_TABLE_PTR,TYPE USER_SOUND  ;Update ptr

CMP     SOUND_TABLE_PTR,OFFSET SOUND_TABLE_END     ;Too big?

JB      APP2                                       ;No

MOV     SOUND_TABLE_PTR,OFFSET SOUND_TABLE_BEGIN   ;Yes, reset

APP2:

PCALLF32_32     WAIT_MSECS_GATE_SEL,0   ;Wait duration

JMP     APP_TASK

APP_CODE32_END:

APP_CODE32  ENDS

;Define stack segments for all privilege levels

; that can be called by this task.

APP_STACK32       SEGMENT

APP_STACK32_BEGIN LABEL WORD

DD      400H     DUP(0)

APP_STACK32_END LABEL WORD

APP_STACK32       ENDS

APP_STACK32_PL1       SEGMENT

APP_STACK32_PL1_BEGIN LABEL WORD

DD      400H     DUP(0)

APP_STACK32_PL1_END LABEL WORD

APP_STACK32_PL1       ENDS

APP_STACK32_PL0       SEGMENT

APP_STACK32_PL0_BEGIN LABEL WORD

DD      400H     DUP(0)

APP_STACK32_PL0_END LABEL WORD

APP_STACK32_PL0       ENDS

# 7	Protected Mode Startup and Termination Module

There is also a task that belongs to the kernel. It serves as the initial task—the first task dispatched by the initialization procedure. It performs as much of system initialization as can be done in protected mode. It also serves as a watchdog task that waits for a prespecified time, then shuts the kernel down. This guarantees termination in spite of possible failures in the primary application task. It performs as much of the kernel shut-down as can be done in protected mode, then jumps back to the 16-bit segment where shut-down will be completed.

## 8	Applying Protection to Kernel Tasks

While it is not necessary, it is often convenient for a kernel to have one or more tasks of its own, and this task is an example of such a situation. We use this task to illustrate two points about tasks in general:

·	The code in a task can execute at any privilege level, not just PL3. The code in this task executes at PL0, because it is part of the kernel. Its code and data segments are at PL0.

·	A task does not necessarily have an LDT. All the segments of this task are located in the GDT. This design decision is arbitrary; the segments could also be in an LDT.

## 9	Example Code

This module consists of:

·	A compile-time variable KERNEL_TIMEOUT that specifies how long the example may run.

·	A task that gets control as soon as the system is in protected mode. The task calls various global initialization procedures, then waits KERNEL_TIMEOUT milliseconds, giving the main tasks of the experiment a chance to run. When it gets control again, it is time to terminate the protected mode example. It simply calls the global termination procedures, then jumps to the real-mode termination procedure.

Example 7.  Kernel TaskTC "7	Kernel Task" \F X \L 2§

;**************** KERNEL TASK module

;It uses two segments: the kernel code and kernel stack.

KERNEL_CODE32  SEGMENT USE32 PUBLIC

ASSUME  CS:KERNEL_CODE32,DS:KWAIT_DATA32,SS:KERNEL_STACK32

CLEAR_SPEAKER_GATES     PROC

;Clears the timer 2 gate (bit 0) and speaker data (bit 1)

PORT_B  EQU     61H     ;I/O port B

IN      AL,PORT_B

AND     AL,11111100B    ;Clear speaker bits

OUT     PORT_B,AL

RET

CLEAR_SPEAKER_GATES     ENDP

KERNEL_TIMEOUT  EQU     25000  ;Units = milliseconds

;The kernel terminates after KERNEL_TIMEOUT milliseconds

KERNEL_WAIT_VALUE       EQU     KERNEL_TIMEOUT*1193/TIMER_VALUE

;The KERNEL_WAIT_VALUE is in units of the clock tick (interrupt period).

;The arithmetic converts from milliseconds to clock tick units.

KERNEL_START32:

CALL    TIMER_INIT              ;Initialize timer

CALL    CLEAR_SPEAKER_GATES     ;Clear bit0 and bit1

CALL    ENABLE_LEVEL_0          ;Enable timer interrupt

STI                             ;Dnable INTR interrupts

MOV     EAX,KERNEL_WAIT_VALUE   ;Get timeout parameter

PCALLF32_32     WAIT_TICKS_GATE_SEL,0        ;Wait

;Here it is time to quit--from one of three sources.

;1 KERNEL timeout

;2 overflow

;3 end of sound table

KERNEL_SHUTDOWN:

CALL    TIMER_RESTORE

PJMPF32_16      RET_CODE16_SEL,RET_REAL  ;Back to real mode

KERNEL_CODE32  ENDS

# 10	Real Mode Startup and Termination Module

At startup, the kernel is running in 16-bit real mode. It has several functions to perform before it can start running the 32-bit, protected-mode application.

1.	Set up the necessary protected-mode structures. In our example, the GDT, IDT, LDT, and TSS structures are defined statically by assembly language declarations. However, because we are using a real-mode loader (COMMAND.COM) that has no idea how to fix up the segment base addresses in protected-mode descriptor tables, we have to fix-up the addresses ourselves.

2.	Enter protected mode.

3.	Switch to the first task.

This example has an additional step to perform at startup: reassign the DOS external interrupts to vectors that do not conflict with protected-mode exceptions. This is necessary because the AT platform uses certain interrupt vectors that are reserved by Intel. These interrupt vectors are assigned to processor exceptions in protected mode.

The shutdown procedure reverses the steps performed by startup procedure.

1.	Return the processor to real mode.

2.	Restore the interrupt vectors.

3.	Return control to DOS.

Table 2 summarizes how the segment and gate descriptors mentioned in previous sections are assigned to privilege levels and descriptor tables. Normally, the base addresses in segment descriptors are initialized through the cooperative efforts of the linker and loader. Because we are using a linker and loader that were not designed for this purpose, the start-up procedure puts the base address for each segment in the GDT or in the appropriate LDT.

Table 2.  Protection StructureTC "2	Protection Structure" \F T \L 2§

| PL | IDT | GDT | Sound Task LDT | Experiment Task LDT |
| --- | --- | --- | --- | --- |
| 0 | TIMER_ISR_GATE | KWAIT_DATA32 KWAIT_CODE32 SOUND_CODE32_G SOUND_DATA32_G KERNEL_CODE32 KERNEL_STACK32 DUMMY_TSS KERNEL_TSS SOUND_TSS APP_TSS SOUND_LDT APP_LDT | SOUND_STACK32_L_PL0 | APP_STACK32_PL0 |
| 1 |  |  | SOUND_CODE32_L SOUND_STACK32_L | APP_STACK32_PL1 |
| 2 |  |  |  |  |
| 3 |  | CHANGE_SOUND_GATE WAIT_TICKS_GATE WAIT_MSECS_GATE |  | APP_CODE32 APP_DATA32 APP_STACK32 |

## 11	Applying Protection to Startup and Termination

Most of this code runs in real mode. For the brief time that it runs in protected mode, it runs at PL0, because the processor always initializes the current privilege level to zero after entering protected mode. Once in protected mode the startup procedure immediately jumps to the initial task, an action that finishes the initialization of protected mode by loading the TR and LDTR registers with functional values.

## 12	Example Code

Example 8 shows a number of assembly language EQU statements that are useful for defining protected-mode data structures. These are used by the startup code.

Example 8.  Protected Mode EquatesTC "8	Protected Mode Equates" \F X \L 2§

;This file contains equate statements that are helpful

;in creating protected-mode data structures.

IO_GATE_SEL   EQU 68H+3

IO_CALL_SEL   EQU 28H

;DESCRIPTOR FIELD VALUES

;Masks for present bit

PRESENT       EQU 80H

NOT_PRESENT   EQU 0

;Masks for descriptor PL field

DPL0          EQU 00H

DPL1          EQU 20H

DPL2          EQU 40H

DPL3          EQU 60H

;Masks for descriptor S bit

SEGMENT_DESC  EQU 10H

SYSTEM_DESC   EQU 0

;Masks for type field of segment descriptors

EO_SEG        EQU 8       ;EXECUTE ONLY

ER_SEG        EQU 0AH     ;EXECUTE AND READ

EOC_SEG       EQU 0CH     ;EXECUTE ONLY, CONFORMING

ERC_SEG       EQU 0EH     ;EXECUTE AND READ, CONFORMING

RO_SEG        EQU 0       ;READ ONLY, NORMAL

RW_SEG        EQU 2       ;READ/WRITE, NORMAL

ROED_SEG      EQU 4       ;READ ONLY, EXPAND-DOWN

RWED_SEG      EQU 6       ;READ/WRITE, EXPAND-DOWN

ED            EQU 4       ;EXPAND DOWN BIT

;Masks for D,B,and G bits of segment descriptors

D_BIT         EQU 40H     ;D BIT FOR 32 BIT TYPES

B_BIT         EQU 40H     ;B BIT FOR 32 BIT TYPE STACKS

G_BIT         EQU 80H     ;G BIT FOR LARGE GRANULARITY

;Type field masks for 32-bit system segments and gates

CALL_G_386    EQU 0CH

INT_G_386     EQU 0EH

TRAP_G_386    EQU 0FH

TSS_AVAIL_386 EQU 09

TSS_BUSY_386  EQU 0BH

LDT_DESC      EQU 2

TASK_G        EQU 5

;Layout of segment descriptor

S_DESC_386      STRUC

ELIMIT            DW 0

EBASE_L           DW 0

EBASE_M           DB 0

EACCESS           DB 0

E386              DB 0

EBASE_H           DB 0

S_DESC_386      ENDS

;Layout of gate descriptor

G_DESC_386      STRUC

OFFSET_LOW        DW 0

SEGMENT_SELECTOR  DW 0

DWORD_COUNT       DB 0

GACCESS           DB 0

OFFSET_HIGH       DW 0

G_DESC_386      ENDS

;Layout of task state segment

TSS_STRUC       STRUC

RBACKLINK         DW 0       ;0 --0

RES1              DW 0       ;1 --2

RESP0             DD 0       ;2 --4

RSS0              DW 0       ;3 --8

RES2              DW 0       ;4 --A

RESP1             DD 0       ;5 --C

RSS1              DW 0       ;6 --10

RES3              DW 0       ;7 --12

RESP2             DD 0       ;8 --14

RSS2              DW 0       ;9 --18

RES4              DW 0       ;10--1A

RCR3              DD 0       ;11--1C

REIP              DD 0       ;12--20

REFLAGS           DD 0       ;13--24

REAX              DD 0       ;14--28

RECX              DD 0       ;15--2C

REDX              DD 0       ;16--30

REBX              DD 0       ;17--34

RESP              DD 0       ;18--38

REBP              DD 0       ;19--3C

RESI              DD 0       ;20--40

REDI              DD 0       ;21--44

RES               DW 0       ;22--48

RES5              DW 0       ;23--4A

RCS               DW 0       ;24--4C

RES6              DW 0       ;25--4E

RSS               DW 0       ;26--50

RES7              DW 0       ;27--52

RDS               DW 0       ;28--54

RES8              DW 0       ;29--56

RFS               DW 0       ;30--58

RES9              DW 0       ;31--5A

RGS               DW 0       ;32--5C

RES10             DW 0       ;33--5E

RLDTR             DW 0       ;34--60

RES11             DW 0       ;35--62

RTBIT             DW 0       ;36--64

RIOBASE           DW 0       ;37--66

TSS_STRUC       ENDS

Example 9 shows a number of assembly language macros that are useful when setting up protected mode data structures. The first of these, LGDTD, SGDTD, LIDTD, and SIDTD, are just 32-bit versions of the assembler's LGDT, SGDT, LIDT, and SIDT instructions. The macro PUT_BASE_IN_DESCRIPTOR tricks the assembler and linker into providing the base address of a segment and moves that address into the split base-address fields of a descriptor. The macro INIT_GDTIDT_ADDRESS stores a segment address in the special memory format required by the LIDT and LGDT instructions.

Example 9.  Protected Mode MacrosTC "9	Protected Mode Macros" \F X \L 2§

LGDTD   MACRO   PARM1

DB      66H

LGDT    PARM1

ENDM

SGDTD   MACRO   PARM1

DB      66H

SGDT    PARM1

ENDM

LIDTD   MACRO   PARM1

DB      66H

LIDT    PARM1

ENDM

SIDTD   MACRO   PARM1

DB      66H

SIDT    PARM1

ENDM

PUT_BASE_IN_DESCRIPTOR MACRO LABEL1,DESCRIPTOR1

;This macro calculates the real-mode linear address of LABEL1

;and stores it in the base address field of DESCRIPTOR1

;We assume the descriptor is in the DS segment at this time.

;Input, AX= real mode segment, ESI points to descriptor.

;EDI = offset to label in real mode segment

MOV     EAX,0

MOV     AX,SEG LABEL1

SHL     EAX,4

MOV     ESI,OFFSET DESCRIPTOR1

ADD     EAX,OFFSET LABEL1         ;add offset of label

MOV     [ESI].S_DESC_386.EBASE_L,AX

SHR     EAX,16

MOV     [ESI].S_DESC_386.EBASE_M,AL     ;NOW BASE IS SET UP

ENDM

INIT_GDTIDT_ADDRESS MACRO LABEL2,BASE_VARIABLE

;The linear address of the GDT/IDT is stored in the appropriate

;variable = BASE_VARIABLE.  We assume the BASE_VARIABLE is in

;the DS segment.

MOV     EAX,0           ;CLEAR EAX

MOV     AX,SEG LABEL2

SHL     EAX,4           ;EAX = BASE OF DATA SEGMENT

ADD     EAX,OFFSET LABEL2

MOV     BASE_VARIABLE,EAX    ;SET UP GDT BASE ADDRESS

ENDM

The real-mode startup code in Example 10 makes heavy use of the equates in Example 8 and macros in Example 9. All the descriptors used in protected mode are partially defined by equates. The entry point is START. It follows these steps:

1.	Put base addresses into the statically defined descriptors.

2.	Load the GDTR and IDTR.

3.	Ready the PIC for protected mode by calling PROTECTED_8259.

4.	Change the processor's mode to protected.

5.	Jump to the first task. The PJMPF16_32 macro is used for this purpose because it uses the correct override prefixes for jumping from a 16-bit segment to a 32-bit TSS.

The label RET_REAL marks the code that changes back to real mode. It follows these steps:

1.	Load segment registers with values appropriate for real mode.

2.	Set the IDTR to point to the real-mode IDT.

3.	Switch to real mode.

4.	Flush the instruction prefetch queue so that addresses previously decoded in protected mode can be decoded again according to real mode rules.

5.	Prepare the PIC for real mode by calling REAL_8259.

6.	Tell DOS that the program is done.